Приложение

| Комманда | Короткая форма | По умолчанию | Параметры | ## Описание |
| --- | --- | --- | --- | --- |
| FIRMRNR | FIR | OFF | ON/OFF | The other station sends a notice (packet) to you if it is not ready to receive data. When ON, receiving such a notice causes the TNC to suspend transmission until it receives a “ready” notice. |
| FLOW | F | ON | ON/OFF | When ON, starting key entry causes the computer to stop displaying received packets. |
| FRACK | FR | 3 | 0 – 250 | Specifies the interval from one transmission until retry of transmission. The unit of the parameter is 1 second. |
| GBAUD | GB | 4800 | 4800/9600 | Selects 4800 or 9600 bps as the transfer rate between the TNC and the GPS receiver. |
| GPSSEND | GPSS | - | 0 – 159 characters | Specifies the content of the data to be output to the GPS receiver; this data is used to program the default settings on the receiver. The output data is not stored in memory. |
| GPSTEXT | GPST | $PNTS | 0 – 6 characters | Specifies the type of a message to be determined by LTEXT. |
| HBAUD | HB | 1200 | 1200/9600 | Selects 1200 or 9600 bps as the transfer rate between packet stations. |
| LOCATION | LOC | EVERY 0 | EVERY/ AFTER n ( n = 0 - 250 ) | If set to EVERY, sends GPS data at intervals of the specified period (n). If set to AFTER, sends GPS data only once after the specified period (n). The unit of n is 10 seconds. |
| LPATH | LPA | GPS | Call1 ( VIA call2, call3, ... call9 ) | Specifies calls signs to send GPS data. Call 1 is the call sign of the destination. Call2 to call9 are call signs of stations to be digipeated through. |
| LTEXT | LT | - | 0 – 159 cсharacters | Specifies the content of a message to be included in GPS data. |
| LTMON | LTM | 0 | 0 – 250 | Specifies the interval for displaying a message determined by LTEXT on the screen; a message appears like a received beacon packet. The unit of the parameter is 1 second. |
| MCOM | MCOM | OFF | ON/OFF | When ON, causes the TNC to also monitor control packets. When OFF, causes it to monitor only information packets. |
| MCON | MC | OFF | ON/OFF | When ON, causes the TNC to monitor other stations while in connection with the target station. |
| MONITOR | M | ON | ON/OFF | When ON, causes the TNC to monitor packets. |
| # MRPT | MR | ON | ON/OFF | When ON, causes the TNC to display the entire digipeat list for monitored packets. |
| MYCALL | MY | NOCALL | 6 characters + SSID | Specifies your call sign. |
| PACLEN | P | 128 | 0 – 255 | Specifies the maximum length of the data portion of a packet. |
| PACTIME | PACT | AFTER 10 | EVERY/ AFTER n ( n = 0~250 ) | If set to EVERY, sends a packet at intervals of the specified period (n). If set to AFTER, sends a packet only once after the specified period of (n). The unit of n is 100 milliseconds. |
| PERSIST | PE | 128 | 128 | Specifies a parameter to calculate probability for the PERSIST/SLOTTIME method. |
| PPERSIST | PP | ON | ON | Causes the TNC to use the PERSIST/SLOTTIME method when ON, or the DWAIT method when OFF. |

43